[image: image3.jpg]Technologies, Ing



[image: image3.jpg]









[image: image4]

[image: image4]Linear RF Amplifier
· Frequency Response: 500-1000 MHz
· Linear Power: 100 watts 
· Saturated Power: 150 watts 
· Gain: 58 dB
Description:
 Designed for linear application in the 500 to 1000 MHz range. This amplifier utilizes RF Power MOSFET devices that provide high gain, wide dynamic range and an excellent 3rd order intercept point. Suggested applications: multi-carrier, pulse, AM & FM modulation.














  


































































Updated:0609
	ELECTRICAL SPECIFICATION @ VDD= 115VAC: Temp.=25°C, 50Ω System

	Parameter
	Symbol
	Min
	Typ
	Max
	Unit

	Operating Frequency
	BW
	500
	
	1000
	MHz

	Power Output Saturated 
	Psat
	
	150
	
	Watt

	Power Output P-1dB 
	P-1dB
	
	100
	
	Watt

	Gain
	G
	54
	58
	
	dB

	Small Signal Gain Flatness
	ΔG
	
	±1
	±1.5
	dB

	Input VSWR
	S11
	
	1.4:1
	1.7:1
	-

	Harmonics @ 100watts
	H
	
	-45
	
	dBc

	Inter-modulation Point

2 Tones, 10W per tone @  900 & 901MHz
	IP3
	
	58
	
	dBm

	Spurious Signals
	dBc
	
	-70
	-60
	dBc

	Operating Voltage
	Vdc
	100
	115
	240
	Volt

	Operating Current @ 100watts
	Amps
	
	
	11
	Amp

	Enable / Disable (shut down pin: gnd=off, open=on)
	ms
	Typical 1ms OFF, 10ms ON.
	ms

	MECHANICAL SPECIFICATION

	Parameter
	Description
	Limits
	Units

	Dimensions
	19” x 17” x 5.25” 
	Max
	Inch

	RF Connectors IN/OUT
	N in, N out
	-
	-

	Control Connector
	BNC/15 pin D-sub
	-
	-

	Cooling
	Fan forced heat-sink
	-
	-

	Weight
	14
	Typ
	lb

	PROTECTIONS

	Thermal Shutdown
	Bi-metal switch set at 70°C with self reset.
	Typ

	Input Overdrive
	+0dBm Max
	Max

	Load VSWR
	4.0:1 up to 100 watts
	Max

	Reverse Polarity Protection
	None
	-

	ENVIRONMENTAL CHARACTERISTICS

	Parameter
	Symbol
	Min
	Typ
	Max
	Units

	Operating Case Temperature
	Tc
	0°C
	
	+50°C
	°C

	Storage Temperature
	Tstg
	-30°C
	
	+100°C
	°C

	Relative humidity non-condensation
	RH
	95
	
	
	%


Response Curve
[image: image5.jpg]Tr1SI1 Log Mag 5.000d6 Raf 000048 [F1]
MU S21 153 Maj 1.000d6/ Rar 57.0048 LTl
62,00
T 50000000 iz 56057 4
2 GO000000 Mz 5534 o
3 70000000 Mz 5.eas de
61,00 |4 00 00000 Mz 57945 o
£ 50000000 Mz 57901 dE
-6 1.0000000 Gz 56622 b
s0.00
.00
.00
s7.00)
.00
ss.00
s6.00
.00
s2.00 A .

T Start 450 MHz TFEW 70 kHz Stop 1.05 GHz K|






Small Signal Frequency Response Curve
[image: image6.jpg]-7 dBm

1S Leg
b <21 133

62,00

Wag 5.000d5/ Ref 000036 [F1]
W33 1.000ds/ ket 57.00d8 [RT]

T
:
:
:

s0.00
.00
se.00
s7.00)
.00
ss.00
s6.00

s.00

0500000
£00.ono0
70000000
£00°00000
50000000
2600000

Wi
Wiz
Wiz
iz
Wiz
4

7am
500
Eae
e
S
Sl

s2.00
(1 Stan 450 MHz

TFBW 70 kHz

=
Stop 1.05 GHz &






Frequency Response Curve @ 100 Watt Output
Response Curve
[image: image7.jpg]LS Log
b <21 153

62,00

Wag 5.000d5/ Ref 000036 [F1]
W33 1.000ds/ Rar 57.00d8 [RT]

T
H
sno0 |3
H
~
s0.00
.00
.00
s7.00)
.00
ss.00

s6.00

.00

0500000
£00.0no0
70000000
£00°00000
50000000
76600000

Wiz
o
Wiz
iz
Wiz
i

Ex
e
ke
£l
517
2543

s2.00
(1 Stan 450 MHz

TFBW 70 ke

=
Stop 1.05 GHz I





Frequency Response Curve @ 150 Watts Output
[image: image8.jpg]MKR & 1.005 MHz

—-41.87 d3

ATTEN 10 dB

~-4.0 dBm

REF

W.g.,m

m._._mﬂz,m..xw —
i
{
i

el

|

i e e

|
i
e

MARKER A

=

dB

1.00@05 MHz
-41.87

B | e

I

WA SB

| |
|
| |

| SR A

e

SNV SO ———

r

SC FC
CORR

SPAN 6.000 MHz

CENTER 900 .500 MHz

SWP 20.0 msec

1
g

30 Ki

VBW

RES BW 30 kHz




Two Tones 10 Watts Avg.  Per Tone @ 900 & 901Mhz IP3 = 61dBm
Response Curve
	[image: image1.emf]HARMONICS @ 100 Watts

-80.00

-70.00

-60.00

-50.00

-40.00

-30.00

-20.00

500 550 600 650 700 750 800 850 900 950 1000

Freq. (MHz)

(dBc)

2nd 3rd

   





	[image: image2.emf]COMPRESSION @ 100 Watts

0.00

0.10

0.20

0.30

0.40

0.50

0.60

0.70

0.80

0.90

1.00

500 550 600 650 700 750 800 850 900 950 1000

Freq.(MHz)

(dB)





Outline Drawing
[image: image9.jpg]i Technologies. Inc

POWER

DCPWR ®

OVER TEMP ®

MODEL # NP-2518
RF INPUT

©

0.dBm MAX

RF POWER AMPLIFIER

500-1000MHz.

RF OUTPUT

100 Watts, 58dB GAIN














2393 Teller Rd. Suite 108









Web:     www.nptrf.com 










Phone: (805) 376-9299

Newbury Park, Ca. 91320









Email: sales@nptrf.com










Fax:     (805) 376-9288

[image: image10.jpg]‘ ) T
—— ;‘?:i‘}@ O o
[
= =] o Y o o
——> 3
== G Q=
18.70" [42.42cm] [=— 622" [13.280m] —
- !— i )
° - %
e e
e
@
o
17.00° [43.18cm] | © - -
-
= ® & e
@ ®

19.00" [48.26cm]

RS W ¥ FOWER ALY W00, W0 Yk, B AN

9 na—
m=_ ® ® [
ame 225°[5.71cm]  5.22" [13.26cm]

: |

s e

[«
S > C 5 ¢
< > C 3 ¢
C

> C 5 ¢




